KUNA RURAL FIRE DISTRICT

INSPECTION CHECKLIST FOR THE SALE AND STORAGE

OF SAFE AND SANE FIREWORKS

Requirements for temporary ﬁreworks sales stand:

- Must be at least 25 feet away from any other building or structure and  from any generator or other internal combustion power sources and/or their fuels.

- Stand must be at least 100 feet away from fuel storage/dispensing station.

- Stand must meet the 2018 IFC requirements for a temporary and permanent tents and membrane structures. (International Fire Code, Chapter 31, Section 3104 and all subsections)

- No temporary fireworks stand shall have a floor area in excess of seven hundred (700) square feet.

- Any and all temporary fireworks stands shall have exit doors at least thirty inches (30") wide at both ends of the temporary fireworks stand and one additional door for each thirty two feet (32') of rear wall in excess of thirty two feet (32'). All doors shall open outward from the temporary fireworks stand and all doorways shall be kept clear and operational at all times. No operator shall cause or allow any exit from a temporary fireworks stand to be obstructed, unopenable, inaccessible, or malfunctioning.

- Each stand/tent shall be provided with a minimum of one (1) type 2A10BC rated fire extinguisher, and one (1) 2A water type fire extinguisher in good working order and with a current inspection tag in place dated within the last calendar year. Extinguishers must be hung three to four feet above the floor. Extinguishers must be readily visible and accessible at all times.

- At least one sign reading, "Fireworks - No Smoking Within 25 Feet" in letters at least four inches (4") high on a contrasting background shall be posted on each and every exterior wall and/or side of any and all temporary fireworks stands. Such signs shall be maintained in a legible condition at all times.

- At least one sign reading, "Fireworks - No Smoking Within 25 Feet" in letters at least two inches (2") high on a contrasting background shall be posted on each and every interior wall and/or side of any and all temporary fireworks stands. Such signs shall be maintained in a legible condition at all times.

- There shall be at least one supervisor twenty-one (21) years of age or older, during business hours. No person under sixteen (16) years of age shall work at or be in the non-public areas of the stand.

- No operator shall sell fireworks to any person who is under the age of sixteen (16) years unless such person is accompanied by a parent or legal guardian. The burden of verifying the age of purchasers of fireworks shall be borne by the operator.

- The stand must be attended during business hours. No person shall sleep at any time in any temporary fireworks stand.

- Waste receptacles or garbage cans in any temporary fireworks stand shall be noncombustible and shall have a securely fitting lid.

- Any and all temporary fireworks stands shall comply with all applicable provisions of this code, including, but not limited to, the unified development code, the building code, the fire code, and the electrical code. The operator shall obtain any and all required permits. No open flame-heating device may be used in a temporary ﬁreworks stand/tent at any time.

- There shall be NO short-term storage within 25’ of ﬁreworks stand/tent. The storage container must be less than 320 square feet and be located at least 25’ from any building or structure. Must have a 2A ﬁre extinguisher located within 48" of the door and be at a height of three to four feet.

- At least one sign reading, "No Fireworks To Be Discharged Within 100 Feet" in letters at least four inches (4") high on a contrasting background shall be posted on every exterior wall or side of all temporary fireworks stands. Such signs shall be maintained in a legible condition at all times.

- Prior to erecting any temporary fireworks stand, and throughout the applicable sales period, the ground underlying such temporary fireworks stand and all ground within a thirty foot (30') perimeter thereof shall be cleared and maintained clear of any and all weeds, combustible vegetation or material, and/or debris. No operator shall permit the accumulation of rubbish or trash in or around any temporary fireworks stand. No operator shall permit a fire nuisance or hazard to exist in or around any temporary fireworks stand.

- All stands shall be 25’ from any generator or internal combustion engine and vehicle parking.

- All permits are required to be posted in the stand at all times.

- Allowed to be erected (2) two temporary signs not to exceed 32 square feet each.  The posting of these signs shall not obscure vehicles line of sight.

- One additional sign may be erected and affixed to the stand.  Sign not to exceed 32 square feet.

- Any and all temporary fireworks stands shall be emptied of all fireworks when not open for business.

No stand will be permitted to open until all requirements are met.